# Lichfield District Council

# Operational Services – Parks and Open Spaces

# Tree Management Procedure

Background

Park trees significantly contribute to the natural green space, in environmental, social, and economic areas in the district.  Removing airborne pollutants, reducing surface water and temperature, being crucial to health and wellbeing.  This Tree Management Procedure has been developed to give clear parameters for the management and maintenance of the council’s tree stock, estimated in the region of 30,000 trees.  As trees are a living dynamic structure, it is not possible to anticipate every situation, however, with this process the council will meet its duty of care, legal, health and safety obligations, ensuring the trees are as safe as reasonably practicable.

# Core procedural points:

The Council will:

- continue to manage its tree stock based on the methodology set out below so that it can ensure its trees are as safe as is reasonably practical.

- continue to train park staff to a basic level of tree inspection so that they’re on hand to carry out inspections whenever this is needed.

- Co-ordinate independent qualified arboricultural inspections in line with the below approach.

- act upon its tree inspections and carry out any works that needs doing within a reasonable timescale.

- treat all complaints and enquiries in the same manner, by discussing, visiting, and inspecting the trees, providing advice and feedback set out in this policy.

- consider completing works to trees where there is a threat to public health/safety, could cause damage to property, or is a conduit for anti-social behaviour.

- consider completing works to trees that are diseased but will take into account national guidance as well as considering the overall impact on works across the district. If diseased trees are found to be a widespread issue, then further advice and guidance will be sought from Cabinet and or a Scrutiny Committee.

- after basic inspections are completed and where there is doubt about the health and safety of a tree, expert inspections will be requested from qualified arboriculturalists.

- after we have inspected a tree, if further action is required, we will identify which category this action fits into.

- carry out visual inspections following Met Office weather warning alerts for high wind, graded amber and above.

- also carry out quick visual inspections to trees when they are in the vicinity of large-scale events in parks.

- trees identified as dangerous, but not within LDC’s remit to be reported to the relevant landowner and backed up with an email/in writing as appropriate.

The Council will not:

- consider completing works if the complaint or enquiry is about leaf fall and litter, overhanging branches, blocking light to gardens, fruit or seeds falling, blocking views, or is causing nuisance because of wildlife, like birds or squirrels, or sap.

- Our approach

In order for us to manage our trees safely, the following methodology has been implemented, considering several factors including: a tree’s size, location, age and susceptibility to recognised diseases.

- Tree Location/Site Risk

The risk of individual areas across the district are in line with the Parks site inspection matrix, as follows:

| High | school run, exercising & dog walking sites, cut through to shops/facilities, play equipment on site |
| --- | --- |
| Medium | site which has, or neighbouring some infrastructure, no play equipment, evidence of desire lines. |
| Low | site with no formal play, little or no infrastructure, not a destination place. |

- Tree Condition (as per Arbortrack category, assigned by inspector)

| Condition | Summary | Risk |
| --- | --- | --- |
| Felled/fallen | The tree has fallen or been felled | No risk |
| Stump | Tree felled but stump still present, potential re-growth. Minimal at time of inspection, potential for change in future years | Low risk |
| Good | Tree showing good crown health, leaves/needles, and trunk. No signs of pests or diseases. | Low risk |
| Reasonable | May be showing signs of dieback, deadwood, swelling or cracks in the trunk, leaning, evidence of fungus, pests and/or cavities. | Medium risk |
| Poor | Crown dieback, fallen limbs, diseased. Evidence of being unstable, new leaning apparent, storm damage. | High Very high risk |
| Dead | Tree shows signs of failure, dropping limbs, of immediate danger | Very high risk |
| - | Young trees less than 75mm diameter (measured 1.5m from the base) and those removed, or which have fallen completely and are to be left for wildlife habitat, to be inspected as part of the Parks Team Site Inspections. | Minimal No risk |

Inspection calculation matrix

|  |  | Tree Condition |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Dead | Poor | Reasonable | Good | Stump | Felled / Fallen |
| Location / Site Risk | High | Immediate Action to remove/ reduce risk Arbortrack re-inspect 6 months, if not removed at time of inspection. | Immediate Action to remove/ reduce risk Arbortrack re-inspect 12 months. | 2 yearly | 3 yearly | 5 yearly | No further expert tree inspections |
|  | Medium | Immediate Action to remove/ reduce risk Arbortrack re-inspect 12 months, if not removed at time of inspection. | Immediate Action to remove/ reduce risk Arbortrack re-inspect 2 yearly | 3 yearly | 4 yearly | 5 yearly | No further expert tree inspections |
|  | Low | Immediate Action to remove/ reduce risk Arbortrack re-inspect 2 years, if not removed at time of inspection. | 3 yearly | 4 yearly | 5 yearly | 5 yearly | No further expert tree inspections |

- Responding to Complaints

The Council will always respond to complaints and queries about trees and there are potentially several responses that we might provide: all being dependant on the tree, its condition, its location, and the nature of the request.

But we recognise that one person’s opinion of a tree might be very different to another persons. In consequence, and given a limited budget, we will not undertake works to trees unless it is necessary to do so.

If we receive complaints or queries, then we follow the process detailed below:

- Check that the tree is LDC’s on Arbortrack and the councils GIS database – if not owned by LDC advise the complainant accordingly. No further action.

- If the tree is LDC’s then confirm its status on Arbortrack and check the following: Is the tree subject to a TPO or is it in a Conservation Area, are any works planned, is it being monitored, has this person made previous enquiries.

- If the complaint or request is about dropping leaves, overhanging branches, blocking light to gardens, causes fruit to fall, blocking views or is causing nuisance because of wildlife or sap then advise the customer that we don’t carry out works to trees for these reasons and refer a note to the Arbortrack schedule.

3a) If the complaint is about over hanging branches and the tree is not protected by a TPO or in a conservation area, we will advise the customer that they are able to complete this work themselves.  We will also advise them if the trees are protected and what procedures they need to follow if they are.

3b) If a complaint continues, we might allow the customer to carry out works to a tree at their own expense as long as the works are specified in advance and agreed with the council. There is no guarantee for this, and the council’s Arboricultural Officers can provide the appropriate technical advice on any specifications put forward.

- If the complaint relates to the poor condition of a tree and which might threaten public health/safety, cause property damage, or is a conduit for anti-social behaviour then the tree will be inspected on a case-by-case basis and referred to the manager. If the manager agrees, then action as necessary. If reported as dangerous and parks staff/managers are uncertain then refer to Arboricultural Officers for a re-inspection.

- Complaints and queries will be logged on Arbortrack against each tree.

- Recording

Works including basic, expert and detailed inspections, removals, complaints and decisions to be recorded on Arbortrack.

Trees planted to be recorded with details as to initial maintenance programme undertaken.

- Budgeting

Any works for trees in parks will either be completed by the qualified in-house team or procured in line with the councils’ financial procedures. The parks team has a limited budget set aside to maintain all its trees and maximising these resources is essential.

There will be times when emergency works are needed and in these circumstances the councils Grounds Maintenance department or a contractor that has provided a day rate for works will be used.

- Tree Software licence

The Software license is paid from the Parks revenue budget.